SOUND GENERATION

Port 43h provides access to the registers of port 42h. First store the magic value 0xb6 to port 43h, then load two 8 bit values into port 42h, which specify the frequency to generate. Once this is done, turning on bits 1 and 0 of port 61h will enable the circuitry and produce the tone. Summarising the steps, they are

1: Output 0xb6 to port 43h

2: Send each of the 8 bit values to port 42h

3: Enable bits 0 and 1 of port 61h

Step 1 is achieved by the C statement outportb( 0x43, 0xb6 );

Step 2 is achieved by converting the frequency into two eight bit values, then outputting them to port 42h. The Most Significant Byte is sent last.

Frequency required to generate = 512 hertz

16 bit value = 120000h / frequency

so, this is achieved by the C statements

outportb(0x42,0)

outportb(0x42,6)

Step 3 is achieved by the C statements

byte = inportb(0x61);

byte |= 3;

outportb(0x61, byte);

Connecting this together into a function, it becomes,

void tone_512() {

char byte;

outportb(0x43, 0xb6);

outportb(0x42, 0);

outportb(0x42, 6);

byte = inportb(0x61);

byte |= 3;

outportb(0x61, byte);

}

There follows two routines to generate sound using the timer chip. The first, beep(), sounds a note of 1000hz for a short duration. The second, note() allows you to specify the frequency and duration of the desired note.

#include <dos.h>

void beep() {

int delay;

_AL = 0xb6;

outportb(0x43,_AL); /* write to timer mode register */

_AX = 0x533; /* divisor for 1000hz */

outportb(0x42,_AL); /* write LSB */

_AL = _AH;

outportb(0x42,_AL); /* write MSB */

_AL = inportb(0x61); /* get current port setting */

_AH = _AL; /* save it in _AH */

_AL |= 3; /* turn speaker on */

outportb(0x61,_AL);

for(delay = 0; delay < 20000; delay++)

;

_AL = _AH;

outportb(0x61,_AL); /* restore original settings */

}

int note( frequency, duration )

unsigned int frequency, duration;

{

unsigned long delay;

if( frequency > 5000 ) return 1;

_AL = 0xb6;

outportb(0x43,_AL); /* write to timer mode register */

_AX = 0x120000L / frequency; /* calculate divisor required */

outportb(0x42,_AL); /* write LSB */

_AL = _AH;

outportb(0x42,_AL); /* write MSB */

_AL = inportb(0x61); /* get current port setting */

_AL |= 3; /* turn speaker on */

outportb(0x61,_AL);

for(delay = 0L; delay < (long) duration * 45; delay++)

;

_AL = inportb(0x61); /* turn off sound */

_AL &= 0xfc; /* set bits 1 and 0 off */

outportb(0x61,_AL); /* restore original settings */

return 0;

}

main() {

unsigned int f;

for(f = 100; f < 250; f += 10 ) {

note( f, (unsigned int) 1000 ); /* 1000 = 1 second */

}

}